# HS017

# HS Risk management form

For additional information refer to HS329 Risk Management Procedure

| Faculty/Division: DCV Research |  |  | School/Unit: X-ray Diffraction Laboratory, Mark Wainwright Analytical Centre |  |  |
| --- | --- | --- | --- | --- | --- |
| Document number RMF-SCXRD-01 | Initial Issue date 27 Nov 2020 | Current version V 3.1 |  | Current Version Issue date 19 Feb 2024 | Next review date 19 Feb 2027 |

| Risk management name | Operation of single-crystal X-ray diffractometer at F10: G63 |
| --- | --- |

| Form completed by |  | signature |  |
| --- | --- | --- | --- |
| Responsible supervisor/ authorising officer | Ruoming Tian | signature |  |

| Identify the activity and the location of the activity |  |
| --- | --- |
| Description of activity | Using X-ray diffractometer to perform Diffraction analysis on single crystal materials, to determine unit cells and crystal structure. |
| Description of location | Room G63, June Griffith Building (F10) |

| Identify who may be at risk from the activity: This may include fellow workers, visitors, contractors and the public. The types of people may affect the risk controls needed and the location may affect the number of people at risk |  |
| --- | --- |
| Persons at risk | XRD instrument users working in the laboratory |
| How they were consulted on the risk | XRD induction and training session |

List legislation, standards, codes of practice, manufacturer’s guidance etc used to determine control measures necessary

Work Health and Safety Act 2011 Work Health and Safety Regulation 2017 NSW radiation control regulation 2013 Fundamentals for protection against ionising radiation (2014) Guide for radiation protection in existing exposure situations (2017)

| Identify hazards and control the risks. 1. An activity may be divided into tasks. For each task identify the hazards and associated risks. Also list the possible scenarios which could sooner or later cause harm. 2. Determine controls necessary based on legislation, codes of practice, Australian standards, manufacturer’s instructions, safety data sheets etc. 3. List existing risk controls and any additional controls that need to be implemented 4. Rate the risk once all controls are in place using the risk rating matrix (below and in HS329 Risk Management Procedure) SHADED GREY AREAS If you need to determine whether it’s reasonably practicable to implement a control based on the risk, complete the shaded grey columns ##### Feel free to resize the boxes to suit your situation/the amount of text you need to use |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Task/ ##### Scenario | ##### Hazard and ##### Associated harm | ##### Existing controls | ##### Any additional controls required? | ##### Risk Rating |  |  | ##### Cost of controls (in terms of time, effort, money) | ##### Is this reasonably practicable ##### Y/N |
|  |  |  |  | ##### Consequences | ##### Likelihood | ##### Risk |  |  |
| ##### Radiation | X-ray radiation is hazardous, with localized exposure capable of causing severe radiation burns. Prolonged exposure increases the risk of leukemia, cancer, and DNA damage. | - The X-ray cabinet is constructed from stainless steel and leaded glass to shield against X-rays from the generator. - The X-ray tube is enclosed within the tube housing. Always check the shutter indicator light before opening the cabinet door. Do NOT open the door if the shutter light is on. - Examine the dose rate monthly. | N | ##### 5 | ##### E | ##### M | ##### No additional cost | ##### Y |
| ##### Cryogenic burns | Skin contact with flowing cold nitrogen steam can cause cold burns. | - Refer to the MSDS for guidelines on liquid nitrogen refilling and transport. - When loading or unloading a sample, set the sample stage to mount position in software. Avoid placing fingers directly in the flow of cold nitrogen steam. | N | ##### 2 | ##### D | ##### L | ##### No additional cost | ##### Y |
| ##### Samples and handling | Crystal samples and solvents may contain hazardous substances. Please list the crystal compositions and commonly used solvents here). | - Please provide control measures here. provide solvent’s MSDS if available. | N | ##### 3 | ##### D | ##### M | ##### No additional cost | ##### Y |
| ##### Lab general activities | Spill or leaks: risk from solvents, acids or bases used in crystallisation. Sharp objects: needles or broken glassware may cause injury | Store all chemicals in appropriately labelled, sealed containers. Use small quantities whenever possible to reduce the risk and impact of spills. Ensure access to spill kits and train staff/users in their proper use. Use tools like tweezers and needles carefully, avoiding excessive force. Dispose of broken glass in designated sharps containers. | N | ##### 2 | ##### D | ##### L | ##### No additional cost | ##### Y |

Risk Rating Matrix

| RISK RATING METHODOLOGY AND MATRIX |  |  |
| --- | --- | --- |
| Consider the Consequences Consider: What type of harm could occur (minor, serious, death)? Is there anything that will influence the severity (e.g. proximity to hazard, person involved in task etc.). How many people are exposed to the hazard? Could one failure lead to other failures? Could a small event escalate? | Consider the Likelihood Consider: How often is the task done? Has an accident happened before (here or at another workplace)? How long are people exposed? How effective are the control measures? Does the environment effect it (e.g. lighting/temperature/pace)? What are people’s behaviours (e.g. stress, panic, deadlines) What people are exposed (e.g. disabled, young workers etc.)? | Calculate the Risk 1.Take the consequences rating and select the correct column 2.Take the likelihood rating and select the correct row 3. Select the risk rating where the two ratings cross on the matrix below. VH = Very high, H = High, M = Medium, L = Low CONSEQUENCES. 1; 2; 3; 4; 5. LIKELIHOOD; A; M; H; H; VH; VH. B; M; M; H; H; VH. C; L; M; H; H; VH. D; L; L; M; M; H. E; L; L; M; M; M. |
| 5. Severe: death or permanent disability to one or more persons 4. Major: hospital admission required 3. Moderate: medical treatment required 2. Minor: first aid required 1. Insignificant: injuries not requiring first aid | A. Almost certain: expected to occur in most circumstances B. Likely: will probably occur in most circumstances C. Possible: might occur occasionally D. Unlikely: could happen at some time E. Rare: may happen only in exceptional circumstances |  |

| Risk level | Required action |
| --- | --- |
| Very high | Act immediately: The proposed task or process activity must not proceed. Steps must be taken to lower the risk level to as low as reasonably practicable using the hierarchy of risk controls |
| High | Act today: The proposed activity can only proceed, provided that: (i) the risk level has been reduced to as low as reasonably practicable using the hierarchy of risk controls and (ii) the risk controls must include those identified in legislation, Australian Standards, Codes of Practice etc. and (iii) the document has been reviewed and approved by the Supervisor and (iv) a Safe Working Procedure or Safe Work Method has been prepared and (v) the supervisor must review and document the effectiveness of the implemented risk controls |
| Medium | Act this week: The proposed task or process can proceed, provided that: (i) the risk level has been reduced to as low as reasonably practicable using the hierarchy of controls and (ii) the document has been reviewed and approved by the Supervisor and (iii) a Safe Working Procedure or Safe Work Method has been prepared. |
| Low | Act this month: Managed by local documented routine procedures which must include application of the hierarchy of controls. |

List emergency procedures and controls List emergency controls for how to deal with fires, spills or exposure to hazardous substances and/or emergency shutdown procedures

1. Emergence button for power shut down 2. Fire distinguisher 3. Chemical spills Kit.

| Implementation |
| --- |
| Additional control measures needed: | Resources required | Responsible person | Date of implementation |
| No |  |  |  |

| REVIEW |
| --- |
| Scheduled review date: |  | March 2024 | March 2027 |
| Are all control measures in place? | Yes |  |  |
| Are controls eliminating or minimising the risk? | Yes |  |  |
| Are there any new problems with the risk? | No |  |  |
| Review by: (name) | Christopher Marjo |  |  |
| Review date: | March 2021 |  |  |

| Risk management name and version number: I have read and understand this risk management form |  |  |
| --- | --- | --- |
| Name | Signature | Date |
|  | You will need to sign here |  |

Acknowledgement of Understanding All persons performing these tasks must sign that they have read and understood the risk management (as described in HS329 Risk Management Procedure). Note: for activities which are low risk or include a large group of people (e.g. open days, BBQ’s, student classes etc), only the persons undertaking the key activities need to sign below. For all others involved in such activities, the information can be covered by other methods including for example a safety briefing, induction, and/or safety information sheet (ensure the method of communicating this information is specified here)